GPS Vehicle tracker

(GPS+GSM+SMS/GPRS)

User Manual

This user manual has been specially designed to guide you through the functions and features of your GPS vehicle tracker.

# Standard accessories:

▶ Power cord

▶ Main Unit

# Optional accessories:

▶ Relay

# Specifications

| Dimension | 65.5*38.5*13.5mm |
| --- | --- |
| Backup Battery | 120mAh / 3.7V |
| Voltage | 9-100VDC |
| Operation Temperature | -20℃-60℃ |
| Humidity | 5% - 95% |
| GSM Frequencies | 850/900/1800/1900 MHz |
| GPRS | Class 12 |
| GPS Channel | 20 |
| GPS Sensitivity | -159dBm |
| Acquisition Sensitivity | -144dBm |
| Position Accuracy | 10m |
| TTFF （Open Sky） | Cold Start: ＜38s |
|  | Warm Start：＜15s |
|  | Hot Start：＜2s |
| GSM/GPS Antenna | Built-in design |
| LED Indicator | GSM-green, GPS-blue, Power-red |
| Data Transmit | TCP, SMS |

- LED Indications

| GPS LED Indicator - Blue |  |  |
| --- | --- | --- |
| Flashing (interval 0.1s) | Searching GPS signal |  |
| Solid blue | GPS fix |  |
| OFF | No GPS fix or initializing |  |

| GSM LED Indicator - Green |  |
| --- | --- |
| Quick flashing (interval 0.1s) | GSM initializing |
| Slow flashing (flash 0.1s every 2s) | Receive GSM signal normally |
| Solid green | Connected to GSM network |
| OFF | No GSM signal |

| Power Status - Red |  |
| --- | --- |
| Flashing (interval 0.1s) | Low battery |
| Slow flashing (interval 0.2s) | Full charge |
| Slow flashing (flash 0.1s every 2s) | Normal operating |
| Solid Red | Charging |
| OFF | Low battery/Power off |

Ignition detection indication: three LED indicators take turns flashing.

# Getting Started

Please follow below instructions for ensuring safe and correct use.

## Install the SIM card

Place the SIM card into the device with the blue-colored side facing down. Note: Make sure there is enough credit on the SIM card. If you will be using the

GPRS function, you should pay attention to the current SIM card GPRS charge.

## Install the device

You need to choose somewhere that it won't be found, because the whole point of

fitting covert GPS vehicle tracker is the secrecy element.

Installation please refers to below picture.

NOTE:

- Any high power devices such as reversing radar, anti-theft device or communication equipment would affect the signal of the device.

- All metallic cases of the windshield will attenuate the signal on the tracking device. It’s simply due to the shielding effects of the metal compound of the case.

## Wiring configuration

| Line No. | Specification | Color | Instruction |
| --- | --- | --- | --- |
|  | MOTOR | Yellow | Connect to relay control line |
|  | ACC | Orange | Connect to ACC ignition |
|  | V- | Black(thick) | Negative side car storage battery. |
|  | V+ | Red(thick) |  |

Notes of the relay wiring

The relay wiring of pump: oil connectors of both ends are a fine white line (85) and a fine yellow line (86). The fine white line (85) is connected to vehicle positive power (+12V). The fine yellow line is connected to the device relay control line.

Cut off the positive connection line of the pump; then connect in series to the relay N.C. contact (thick green line 87a) and the other end to relay COM contact (thick green line 30).

## Device wiring diagram